- Program to generate the following discrete time signals. a) Unit sample sequence, b)Unit step sequence, c) Exponential sequence, d)Sinusoidal sequence, e) Random sequence

clc; clear all; close all;

N = 10;

n = -N:1:N;

% unit Impulse

x1=[zeros(1,N) 1 zeros(1,N)];

subplot(4,2,1); stem(n,x1); title(' Impulse Signal');

%unit Step signal

x2=[zeros(1,N) 1 ones(1,N)];

subplot(4,2,2); stem(n,x2); title('Unit step signal');

%Exponential growing/decaying signal

n2=0:0.1:N;

a2=2;

x4=a2.^n2;

subplot(4,2,3); stem(n2,x4); title( 'Exponential growing signal');

a3=0.5;

x5=a3.^n2;

subplot(4,2,4); stem(n2,x5); title( 'Exponential decaying signal');

%cosine signal

x6=cos(n2);

subplot(4,2,5); stem(n2,x6); title('Cosine signal');

%sine signal

x7=sin(n2);

subplot(4,2,6); stem(n2,x7); title('sine signal');

% random sequence

x8=rand(1, 10)

subplot(4,2,7);

hist(x8);title('random sequence');

- Program to perform the following operations on signals. a) Signal addition, b) Signal multiplication, c)Scaling, d) Shifting, e)Folding

x=[1 2 1 2];

subplot (3,2,1); stem(x);title('input x');

y=[2 2 1 1];

subplot (3,2,2); stem(y);title('input y');

%addition

a=x+y;

subplot (3,2,3); stem(a);title('addition');

%multiplication

b=x.*y;

subplot (3,2,4); stem(b);title('multilplication');

clc;close all; clear all;

%folding a signal

clc;close all; clear all;

n=-1:2;

x=[3 -1 0 -4];

subplot(2,1,1);stem(n,x);axis([-3 3 -5 5]);

title('Signal x(n)');

c=fliplr(x);

y=fliplr(-n);

subplot(2,1,2);stem(y,c);axis([-3 3 -5 5]);

title('Reversed Signal x(-n)') ;

%shifting signal

n1=input('Enter the amount to be delayed');

n2=input('Enter the amount to be advanced');

n=-2:2;

x=[-2 3 0 1 5];

subplot(3,1,1);stem(n,x);title('Signal x(n)');

m=n+n1;

y=x;

subplot(3,1,2);stem(m,y);title('Delayed signal x(n-n1)');

t=n-n2;

z=x;

subplot(3,1,3);stem(t,z);title('Advanced signal x(n+n2)');

clc;close all; clear all;

%scaling

n=[-2 -1 0 1 2];

x=[3 1 0 4 6];

subplot(3,1,1);stem(n,x);title('original');hold on; %original

n2 = 4*n;

subplot(3,1,2);stem(n2,x);title('scaled by 4') %x4

n3 = 2*n;

subplot(3,1,3);stem(n3,x);title ('scaled by 2')%x2

- Program to perform convolution of two given sequences (without using built-in function) and display the signals.

clc;close all;

%inputs

x=input('enter the input sequence of x(n)');

h=input('enter the input sequence of h(n)');

%find the length of signal

n1=length(x);

n2=length(h);

%find length of y(n)

N=n1+n2-1;

%zero padding to make the length=N

x=[x,zeros(1,N-n1)];

h=[h,zeros(1,N-n2)];

%initialse the output with zero

y=zeros(1,N);

%perform convolution

for n=1:N

for k=1:n

y(n)=y(n)+x(k).*h(n-k+1);

end

end

disp(y);

%plot the inputs & outputs

ny=0:N-1;

subplot(3,1,1);stem(ny,x);title('first sequence');

subplot(3,1,2);stem(ny,h);title('second sequence');

subplot(3,1,3);stem(ny,y);title('Convoluted sequence');

- Consider a causal system y(n) = 0.9y(n-1)+x(n).

a) Determine H(z) and sketch its pole zero plot.

b) Plot |H(ejω)| and ∠ H(ejω)

c) Determine the impulse response h(n).

% Part A: Transfer Function and Pole-Zero Plot

numerator=1;	% z

denominator=[1,-0.9]; % z - 0.9

H=tf(numerator,denominator);

% Pole-Zero plot

figure;

pzmap(H);

title('Pole-Zero Plot of H(z)');

grid on;

% Part B: Frequency Response

omega = linspace(-pi, pi, 512); % Frequency from -pi to pi

H_freq = freqz(numerator, denominator, omega);

magnitude = abs(H_freq);

phase = angle(H_freq);

% Plot Magnitude

figure;

subplot(2,1,1);

plot(omega, magnitude);

title('Magnitude |H(e^{j\omega})|');

xlabel('\omega (radians/sample)');

ylabel('|H(e^{j\omega})|');

grid on;

% Plot Phase

subplot(2,1,2); plot(omega, phase);

title('Phase \angle H(e^{j\omega})');

xlabel('\omega (radians/sample)');

ylabel('\angle H(e^{j\omega}) (radians)');

grid on;

% Part C: Impulse Response

n= 0:20; % Sample range

h = (0.9).^n; % Impulse response for n >= 0

% Plot Impulse Response figure;

stem(n,h,'filled');

title('Impulse Response h(n)');

xlabel('n');

ylabel('h(n)');

grid on;

- Computation of N point DFT of a given sequence (without using built-in function) and to plot the magnitude and phase spectrum.

% Define the input sequence

x = [1, 2, 3, 4]; % Example sequence

N = length(x);	% Number of points in DFT

% Initialize the DFT output

X = zeros(1, N); % DFT output array

% Compute the N-point DFT for k = 0:N-1

for k = 0:N-1

for n = 0:N-1

X(k+1) = X(k+1) + x(n+1) * exp(-1j * 2 * pi * k * n / N);

end

end

% Magnitude and Phase Spectrum

magnitude = abs(X);

phase = angle(X);

% Frequency bins for plotting

frequencies = (0:N-1) * (2 * pi / N); % Frequency range

% Plotting the Magnitude Spectrum figure;

subplot(2, 1, 1);

stem(frequencies, magnitude, 'filled'); title('Magnitude Spectrum'); xlabel('Frequency (radians/sample)'); ylabel('|X(k)|');

grid on;

% Plotting the Phase Spectrum

subplot(2, 1, 2); stem(frequencies, phase, 'filled'); title('Phase Spectrum');

xlabel('Frequency (radians/sample)'); ylabel('\angle X(k) (radians)');

grid on;

% Adjust layout

sgtitle('N-point DFT of the Sequence');

- Using the DFT and IDFT, compute the following for any two given sequences a)Circular convolution b) Linear convolution

% Define two example sequences

x = [1, 2, 3];  % First sequence

h = [0.5, 1]; % Second sequence

% Lengths of the sequences

N = max(length(x), length(h)); % Length for DFT (circular convolution)

L = length(x) + length(h) - 1; % Length for linear convolution

% Circular Convolution using DFT and IDFT

X = fft(x, N); % DFT of x

H = fft(h, N); % DFT of h

% Circular convolution result

Y_circular = X .* H; % Element-wise multiplication in frequency domain

y_circular = ifft(Y_circular); % IDFT to get the circular convolution result

% Linear Convolution using DFT and IDFT

% Zero-pad the sequences to length L

x_padded = [x,zeros(1,L-length(x))]; % Zero-pad x

h_padded = [h,zeros(1,L-length(h))]; % Zero-pad h

% DFT of the zero-padded sequences

X_linear = fft(x_padded, L); % DFT of x padded

H_linear = fft(h_padded, L); % DFT of h padded

% Linear convolution result

Y_linear = X_linear .* H_linear; % Element-wise multiplication in frequency domain

y_linear = ifft(Y_linear); % IDFT to get the linear convolution result

% Display Results

disp('Circular Convolution Result:'); disp(y_circular);

disp('Linear Convolution Result:'); disp(y_linear);

Circular Convolution Result:

3.5000    2.0000    3.5000

Linear Convolution Result:

0.5000    2.0000    3.5000    3.0000

- Verification of Linearity property, circular time shift property & circular frequency shift property of DFT.

% Define parameters

N = 8; % Length of DFT

n = 0:N-1; % Sample indices

% Define two sequences for testing

x1 = [1, 2, 3, 4, 0, 0, 0, 0]; % First sequence

x2 = [0, 1, 0, 1, 0, 0, 0, 0]; % Second sequence

% Define coefficients for linearity

alpha = 2; % Coefficient for x1

beta = 3; % Coefficient for x2

% 1. Linearity Property

% DFT of individual sequences

X1 = fft(x1);

X2 = fft(x2);

% DFT of the linear combination

X_linear_combination = fft(alpha * x1 + beta * x2);

% Verify linearity

X_combined = alpha * X1 + beta * X2;

% Display results

disp('Linearity Verification:');

disp('DFT of linear combination:'); disp(X_linear_combination); disp('Linear combination of DFTs:'); disp(X_combined);

% 2. Circular Time Shift Property

% Time shift by 2 samples

shift = 2;

x_shifted = circshift(x1, shift); % Circularly shift x1 by 2

X_shifted = fft(x_shifted); % DFT of the shifted sequence

X_original = fft(x1); % DFT of the original sequence

% Verify circular time shift property

% Expected: X_shifted = X_original * e^(-j*2*pi*shift/N*n)

% Display results

disp('Circular Time Shift Verification:'); disp('DFT of shifted sequence:'); disp(X_shifted);

disp('Expected result:');

expected_shifted = X_original .* exp(-1j*2*pi*shift/N * n); disp(expected_shifted);

% 3. Circular Frequency Shift Property

% Frequency shift by 1 sample

freq_shift = 1;

X_freq_shifted = circshift(X_original,freq_shift);

% Circularly shift DFT

% Verify circular frequency shift property

% Expected: x[n] * e^(j*2*pi*freq_shift*n/N)

% Display results

disp('Circular Frequency Shift Verification:'); disp('DFT of frequency shifted sequence:'); disp(X_freq_shifted);

disp('Expected result:');

expected_freq_shifted = X_original .* exp(1j * 2 * pi * freq_shift * n / N); disp(expected_freq_shifted);

% Plotting results for verification figure;

% Linearity Property

subplot(3, 1, 1);

stem(n, real(X_linear_combination), 'filled', 'DisplayName', 'Linear Combination'); hold on;

stem(n, real(X_combined), 'filled', 'DisplayName', 'Combined DFT'); title('Linearity Property Verification');

xlabel('k'); ylabel('Magnitude'); legend;

grid on;

% Circular Time Shift Property

subplot(3, 1, 2);

stem(n, real(X_shifted), 'filled', 'DisplayName', 'Shifted DFT'); hold on;

stem(n, real(expected_shifted), 'filled', 'DisplayName', 'Expected DFT'); title('Circular Time Shift Property Verification');

xlabel('k');

ylabel('Magnitude'); legend;

grid on;

% Circular Frequency Shift Property

subplot(3, 1, 3);

stem(n, real(X_freq_shifted), 'filled', 'DisplayName', 'Frequency Shifted DFT'); hold on;

stem(n, real(expected_freq_shifted), 'filled', 'DisplayName', 'Expected DFT'); title('Circular Frequency Shift Property Verification');

xlabel('k'); ylabel('Magnitude'); legend;

grid on;

sgtitle('Verification of DFT Properties');

- Develop decimation in time radix-2 FFT algorithm without using built-in functions.

function X = radix2_fft(x)

% Check if the length of x is a power of 2 N = length(x);

if mod(N, 2) ~= 0 || N == 0 || (N & (N - 1)) ~= 0 error('Length of input must be a power of 2');

end

% Recursive FFT computation if N == 1

X = x; % Base case return;

end

% Split into even and odd parts

X_even = radix2_fft(x(1:2:N)); % FFT of even-indexed elements X_odd = radix2_fft(x(2:2:N)); % FFT of odd-indexed elements

% Combine results X = zeros(1, N); for k = 0:N/2-1

twiddle_factor = exp(-1j * 2 * pi * k / N) .* X_odd(k + 1); % Twiddle factor X(k + 1) = X_even(k + 1) + twiddle_factor; % Combine even and odd

X(k + N/2) = X_even(k + 1) - twiddle_factor; % Combine even and odd end

end

% Example Usage

N = 8; % Length of the input signal (must be a power of 2) x = [1, 2, 3, 4, 0, 0, 0, 0]; % Example input sequence

% Compute the FFT X = radix2_fft(x);

% Display results disp('FFT Result:'); disp(X);

% Plot the magnitude and phase of the FFT figure;

subplot(2, 1, 1);

stem(0:N-1, abs(X), 'filled'); title('Magnitude of FFT'); xlabel('Frequency Index');

ylabel('|X(k)|'); grid on;

subplot(2, 1, 2);

stem(0:N-1, angle(X), 'filled'); title('Phase of FFT'); xlabel('Frequency Index'); ylabel('\angle X(k) (radians)'); grid on;

sgtitle('Radix-2 FFT Result');

- Design and implementation of digital low pass FIR filter using a window to meet the given specifications.

% Filter specifications

Fs = 1000;

% Sampling frequency (Hz)

Fc = 150;

N = 21;

% Cutoff frequency (Hz)

% Filter order (must be odd)

window_type = 'hamming'; % Type of window ('hamming', 'hanning', 'rectangular')

% Normalize the cutoff frequency

Wn = Fc / (Fs / 2);  % Normalized cutoff frequency (0 to 1)

% Design the ideal low-pass filter (sinc function)

n = 0:N-1;

% Time index

h_ideal = 2 * Wn * sinc(2 * Wn * (n - (N-1)/2)); % Ideal filter impulse response

% Apply window to the ideal filter

if strcmp(window_type, 'hamming')

w = hamming(N)';

elseif strcmp(window_type, 'hanning')

w = hanning(N)';

elseif strcmp(window_type, 'rectangular')

w = ones(1, N);

else

error('Unsupported window type. Use hamming, hanning, or rectangular.');

end

h = h_ideal .* w;  % Apply window

% Normalize the filter coefficients

h = h / sum(h);

% Normalize to ensure unity gain at DC

% Frequency response of the filter

[H, f] = freqz(h, 1, 1024, 'whole');  % Frequency response

% Plot the filter coefficients

figure;

subplot(2, 1, 1);

stem(0:N-1, h, 'filled');

title('FIR Filter Coefficients');

xlabel('n (samples)');

ylabel('h[n]');

grid on;

% Plot the frequency response

subplot(2, 1, 2);

plot(f * Fs / (2 * pi), 20*log10(abs(H))); % Convert to dB

title('Frequency Response of the FIR Filter');

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

grid on;

% Add a vertical line at the cutoff frequency

hold on;

xline(Fc, '--r', 'Cutoff Frequency');

hold off;

sgtitle('Low-Pass FIR Filter Design Using Window Method');

% Optional: Test the filter with a sample signal

t = 0:1/Fs:1;

% Time vector

signal = sin(2*pi*50*t) + sin(2*pi*200*t); % Example signal (50 Hz + 200 Hz)

filtered_signal = filter(h, 1, signal);    % Apply filter

% Plot the original and filtered signal

figure;

subplot(2, 1, 1);

plot(t, signal);

title('Original Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(2, 1, 2);

plot(t, filtered_signal);

title('Filtered Signal (Low-Pass)');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

sgtitle('Signal Filtering using FIR Low-Pass Filter');

- Design and implementation of digital High pass FIR filter using a window to meet the given specifications.

% Filter specifications

Fs = 1000;

% Sampling frequency (Hz)

Fc = 150;

N = 21;

% Cutoff frequency (Hz)

% Filter order (must be odd)

window_type = 'hamming'; % Type of window ('hamming', 'hanning', 'rectangular')

% Normalize the cutoff frequency

Wn = Fc / (Fs / 2);  % Normalized cutoff frequency (0 to 1)

% Design the ideal high-pass filter (using sinc function)

n = 0:N-1;

% Time index

h_ideal = -2 * Wn * sinc(2 * Wn * (n - (N-1)/2)); % Ideal filter impulse response

h_ideal((N-1)/2 + 1) = h_ideal((N-1)/2 + 1) + 1; % Correct the DC gain

% Apply window to the ideal filter

if strcmp(window_type, 'hamming')

w = hamming(N)';

elseif strcmp(window_type, 'hanning')

w = hanning(N)';

elseif strcmp(window_type, 'rectangular')

w = ones(1, N);

else

error('Unsupported window type. Use hamming, hanning, or rectangular.');

end

h = h_ideal .* w;  % Apply window

% Normalize the filter coefficients

h = h / sum(w);

% Normalize to ensure unity gain at high frequencies

% Frequency response of the filter

[H, f] = freqz(h, 1, 1024, 'whole');  % Frequency response

% Plot the filter coefficients

figure;

subplot(2, 1, 1);

stem(0:N-1, h, 'filled');

title('FIR High-Pass Filter Coefficients');

xlabel('n (samples)');

ylabel('h[n]');

grid on;

% Plot the frequency response

subplot(2, 1, 2);

plot(f * Fs / (2 * pi), 20*log10(abs(H))); % Convert to dB

title('Frequency Response of the FIR High-Pass Filter');

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

grid on;

% Add a vertical line at the cutoff frequency

hold on;

xline(Fc, '--r', 'Cutoff Frequency');

hold off;

sgtitle('High-Pass FIR Filter Design Using Window Method');

% Optional: Test the filter with a sample signal

t = 0:1/Fs:1;

% Time vector

signal = sin(2*pi*50*t) + sin(2*pi*200*t); % Example signal (50 Hz + 200 Hz)

filtered_signal = filter(h, 1, signal);    % Apply filter

% Plot the original and filtered signal

figure;

subplot(2, 1, 1);

plot(t, signal);

title('Original Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(2, 1, 2);

plot(t, filtered_signal);

title('Filtered Signal (High-Pass)');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

sgtitle('Signal Filtering using FIR High-Pass Filter');

- Design and implementation of digital IIR Butterworth low pass filter to meet the given specifications.

% Filter specifications

Fs = 1000;

% Sampling frequency (Hz)

Fc = 150;

% Cutoff frequency (Hz)

order = 4;

% Order of the Butterworth filter

% Normalize the cutoff frequency

Wn = Fc / (Fs / 2);  % Normalized cutoff frequency (0 to 1)

% Design the Butterworth low-pass filter

[b, a] = butter(order, Wn, 'low'); % Get filter coefficients

% Frequency response of the filter

[H, f] = freqz(b, a, 1024, 'whole');  % Frequency response

% Plot the filter coefficients

figure;

subplot(2, 1, 1);

zplane(b, a);  % Zero-pole plot

title('Zero-Pole Plot of Butterworth Low-Pass Filter');

% Plot the frequency response

subplot(2, 1, 2);

plot(f * Fs / (2 * pi), 20*log10(abs(H))); % Convert to dB

title('Frequency Response of Butterworth Low-Pass Filter');

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

grid on;

% Add a vertical line at the cutoff frequency

hold on;

xline(Fc, '--r', 'Cutoff Frequency');

hold off;

sgtitle('Butterworth Low-Pass Filter Design');

% Optional: Test the filter with a sample signal

t = 0:1/Fs:1;

% Time vector

signal = sin(2*pi*50*t) + sin(2*pi*200*t); % Example signal (50 Hz + 200 Hz)

filtered_signal = filter(b, a, signal);    % Apply filter

% Plot the original and filtered signal

figure;

subplot(2, 1, 1);

plot(t, signal);

title('Original Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(2, 1, 2);

plot(t, filtered_signal);

title('Filtered Signal (Butterworth Low-Pass)');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

sgtitle('Signal Filtering using Butterworth Low-Pass Filter');

- Design and implementation of digital IIR Butterworth High pass filter to meet the given specifications.

% Filter specifications

Fs = 1000;

% Sampling frequency (Hz)

Fc = 150;

order = 4;

% Cutoff frequency (Hz)

% Order of the Butterworth filter

% Normalize the cutoff frequency

Wn = Fc / (Fs / 2);  % Normalized cutoff frequency (0 to 1)

% Design the Butterworth high-pass filter

[b, a] = butter(order, Wn, 'high'); % Get filter coefficients

% Frequency response of the filter

[H, f] = freqz(b, a, 1024, 'whole');  % Frequency response

% Plot the filter coefficients

figure;

subplot(2, 1, 1);

zplane(b, a);  % Zero-pole plot

title('Zero-Pole Plot of Butterworth High-Pass Filter');

% Plot the frequency response

subplot(2, 1, 2);

plot(f * Fs / (2 * pi), 20*log10(abs(H))); % Convert to dB

title('Frequency Response of Butterworth High-Pass Filter');

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

grid on;

% Add a vertical line at the cutoff frequency

hold on;

xline(Fc, '--r', 'Cutoff Frequency');

hold off;

sgtitle('Butterworth High-Pass Filter Design');

% Optional: Test the filter with a sample signal

t = 0:1/Fs:1;

% Time vector

signal = sin(2*pi*50*t) + sin(2*pi*200*t); % Example signal (50 Hz + 200 Hz)

filtered_signal = filter(b, a, signal);    % Apply filter

% Plot the original and filtered signal

figure;

subplot(2, 1, 1);

plot(t, signal);

title('Original Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(2, 1, 2);

plot(t, filtered_signal);

title('Filtered Signal (Butterworth High-Pass)');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

sgtitle('Signal Filtering using Butterworth High-Pass Filter');